Frequency, Intensity, and Continuity of Dust Storms in Tigris and Euphrates Basin: A MODIS AOD-Based Survey

Mohsen Bakhtiariab, Najmeh Neysani Samanya, Ali Al-Hemoudc, Ali Darvishi Boloorania

a Department of Remote Sensing and GIS, Faculty of Geography, University of Tehran, Tehran, Iran

b General Department of Natural Resources and Watershed Management of Alborz Province, Karaj, Iran

b Environment and Life Sciences Research Center, Kuwait Institute for Scientific Research, Kuwait

Abstract: Dust storms, predominantly in arid and semi-arid regions, exert substantial impacts on climatic conditions, soil, vegetation cover, and the populations' health. This study explores the frequency, intensity, and continuity (FIC) of dust storm events (DSEs) in the Tigris and Euphrates basin (TEB) utilizing the MODIS-Aerosol Optical Depth (AOD). Given the hydro-political and climatic significance of TEB, understanding the spatiotemporal dynamics of DSEs is crucial. The study applies Otsu’s thresholding and weighting average (WA) methods to identify dusty days from blue and green AOD time series for 22 years (2001 to 2022). Dust hotspot sources identified through visual interpretation, serve for calibrating AOD thresholds. The study further employs the K-means clustering method to categorize the intensity and continuity of DSEs and uses Poisson probability distribution to model the probability of dust occurrence. The AOD thresholds assessed by WA for AOD-green bands had higher accuracy. The probability distribution of the DSE frequency revealed a normal distribution. Findings reveal a higher occurrence of severe and continuous DSEs in the central and southern regions of TEB. The derived Dust Hazard Index (DHI) assessed by the combination of FIC of DSEs highlights areas with high dust hazards, emphasizing the need for effective dust mitigation strategies.

Key words: AOD, Probability of dust occurrence, Dust Hazard Index (DHI), Tigris and Euphrates basin (TEB)

- Introduction

Dust storms are meteorological phenomena with their destructive effects that occur in vast regions of the arid and semi-arid lands mainly located in subtropical areas (Wu, 2007). They are considered as the consequence of land degradation (Bakhtiari et al., 2021) and wind erosion (Al-Hemoud et al., 2019). Interaction of wind, bare soil, and dry air situations facilitates the occurrence of dust storm phenomena (Marchese et al., 2017; Jebali et al., 2021), triggering the emission of dust particles into the atmosphere which can travel thousands of kilometers and reduces horizontal visibility to less than 1 km beyond their source areas (Furman, 2003; Al-Hemoud et al., 2022). The dust storm has substantial impacts on climate, air quality, soil properties, and the health of affected regions (Middleton, 2017; Middleton and Al-Hemoud 2024). Climate change and anthropogenic activities have escalated the dust storm event (DSE) frequencies and their impacts in many arid and semi-arid areas (Middleton and Goudie, 2001; Mehrabi et al., 2015; Darvishi Boloorani et al., 2023c). Dust particles have distressing impacts on environment and living organisms including the health of human societies, agricultural products, and live stocks (Soleimani et al., 2020; Aghababaeian et al., 2021), reduce the visibility, disrupts the activity of the transportation systems (Deng et al. 2011), and aggravate air pollution (Guo and Chen, 2016), even in areas far from the emission sources (Middleton, 2017). Estimates show approximately two billion tons of dust particles are annually transmitted mostly through DSEs into the atmosphere (Ginoux et al., 2001). DSE detection, dust particles composition, and dust source region identification are of significant importance to predict and deal effectively with the destructive consequences of dust storms.

Various types of remote sensing (Prasad & Singh, 2007; Kaskaoutis et al., 2008; Liu et al., 2011; Bakhtiari et al., 2018; Boloorani et al., 2023a) and meteorological stationary data (Qian et al., 2002; McTainsh et al., 2005; Namdari et al., 2018; Beyranvand et al, 2023) have been widely used in DSE studies. Aerosol Optical Depth (AOD) has been defined as the quantity of aerosol particles that prevent the transmission of light through the atmosphere (Gupta et al., 2018). The Moderate Resolution Imaging Spectroradiometer (MODIS) sensors mounted on the Terra and Aqua satellites have been widely used for dust studies using the AOD retrieval algorithms such as Dark Target (DT) and Deep Blue (DB) (Wei et al., 2019). DT algorithm retrieves AOD over both land and oceans. If the atmospheric aerosols between the surface and the top of the atmosphere are dense, it works better (Gupta et al., 2018). DB is specially intended to retrieve AOD over bright land surfaces, such as deserts, where the DT algorithm has lower accuracy (Song et al., 2021). The MODIS collection aerosol products have been developed based on the main improvements in both radiometric calibration and aerosol retrieval algorithms (Wei et al., 2019). The AOD retrieval process of MODIS identifies dust from non-dust aerosols based on particle shape information for Cloud-Aerosol Lidar with Orthogonal Polarization (CALIOP), dust size, and absorption information, e.g., fine-mode fraction, Ångström exponent, and single-scattering albedo (Song et al., 2021). The accuracy of the AOD retrievals is validated by comparing MODIS AOD data with ground-based measurements from the AErosol RObotic NETwork (AERONET) (Tripathi et al., 2005).

Albeit, global dust sources include the Sahara, Middle East, Central and Eastern Asia, and some central regions in Australia are mainly in arid and semi-arid regions, but the DSE occurs across and beyond arid and semi-arid lands in any season with various frequencies and intensities (Middleton, 2017). Transboundary Tigris and Euphrates basin (TEB) in west Asia due to its special hydro-political and climatic conditions (Al-Ansari, 2016), has been considered a region with numerous dust sources and DSEs (Darvishi Boloorani et al., 2021). The destructive effects of DSEs within TEB even reach thousands of kilometers away from the dust sources (Al-Hemoud et al. 2020). Although, various aspects and driving factors of DSEs in TEB have been studied in recent decades (Hamidi et al., 2013, Sotoudeheian et al., 2016; Darvishi Boloorani et al., 2020; Darvishi Boloorani et al., 2021; Naghibi et al., 2024), but the spatiotemporal pattern of DSEs has not been thoroughly investigated.

Precise definition and calculation of frequency, intensity, and continuity (FIC) of DSEs could illustrate the spatiotemporal behavior and the probability of the DSE hazard. The main contributions of this paper are suggesting an AOD thresholding approach to identify the DSEs, mapping the spatial-temporal behavior of the dust FIC and developing dust hazard index, and estimating the probability of DSEs in TEB. To achieve this, the study seeks (i) how DSEs are identified and monitored from daily time series of AOD and remote sensing-based visual interpretation datasets; (ii) how the spatiotemporal behaviors of the dust FIC are defined and extracted from the daily time series of AOD?; (iii) how the extracted FIC can be used to assess the probability of DSEs?; and (iv) what is the relationship between dust hazard and dust FIC?

- Materials and methods

  - Study area

Tigris and Euphrates basin is shared between Iraq, Turkey, and Syria (Fig. 1). Throughout this region, different climatic conditions prevail as a function of topographic variability and geographical extent. The altitude ranges from sea level up to 4000 m in the monotonous north and northeast regions. Precipitation and temperature regimes vary mainly due to the elevation and latitude changes. The population is concentrated mostly around the lands of the Tigris and Euphrates rivers. DSEs are governed by the interaction of hydrological and climatic factors, hence the drought periods and water control policies have increased the dust storms in the TEB during the last two decades (Al-Ansari et al., 2016; Darvishi Boloorani et al., 2020; Naghibi et al., 2024).

Fig. 1. The Tigris and Euphrates basin. Dust storm events obtained from https://data.mendeley.com/datasets/7937gn7g8c/1 (Darvishi Boloorani et al., 2023b).

  - Dataset

To identify and monitor daily DSEs and extract spatiotemporal indicators of DSEs, two main data sources were utilized, including the daily time series of AOD and hotspot dust sources (https://data.mendeley.com/datasets/7937gn7g8c/1).

AOD time series: Daily MODIS-AOD were utilized to explore and cluster DSEs. MODIS aerosol DT and DB algorithms retrieve AOD at spectral blue (470 nm) and green (550 nm) bands. The 550 nm band is the most common AOD band used (Romano et al., 2013; Xie et al., 2017). To make a better choice, we compared both blue and green bands to select the more accurate AOD. Daily time series of MCD19A2.061 products with 1 km resolution including blue and green bands were prepared and provided in Google Earth Engine (GEE) in the 2001-2022 period.

Dust hotspots: We used hotspot dust sources identified by visual interpretation of MODIS satellite images in TEB (https://data.mendeley.com/datasets/7937gn7g8c/1) (Darvishi Boloorani et al., 2023b). The dust hotspots have been identified based on dust storm events; hence in this study, they served as benchmark dust events to set the AOD threshold to differentiate between dusty and non-dusty days. Dust hotspots are point locations and each indicates a unique DSE. Dust hotspots are characterized by the date (year/month/day) and coordinates of the observed dust storms (x, y).

  - Methodology

Daily AOD measurements from 2001 to 2022, along with the identified dust hotspots of the DSEs, were utilized to identify daily dusty pixels of 1x1 km resolution. As shown in Fig. 2, the DSEs were entered into Otsu’s thresholding and weighting average (WA) methods to find the annual threshold of AODs identifying dusty days. 70% of randomly selected DSEs were used to assess AOD annual thresholds. The accuracy of assessed AOD thresholds for both methods was measured using the 30% remaining DSE points. Annual AOD thresholds with more accuracy were used to calculate the annual frequency, intensity, and continuity of daily dust pixels.

Fig. 2. Flowchart of the study methodology.

AOD Thresholding: AOD time series do not reveal dust storms explicitly, but are one of the most reliable data sources to monitor the DSEs (Kaskaoutis et al., 2008; Xie et al., 2017).  We used Otsu’s thresholding and WA methods to find the accurate AOD threshold separating dusty days from non-dusty days in the TEB.

Otsu’s method is used for image auto-thresholding, which is the process of converting a grayscale image into a binary image. It separates an image into two classes, based on the grayscale intensity values of its pixels. Otsu uses the grayscale histogram of an image to detect an optimal threshold value that separates two regions with maximum inter-class variance. In other words, the method processes the image histogram, segmenting the objects by minimization of the variance on each of the classes (Xu et al., 2011; Yang et al., 2020). We used Otsu’s method to discriminate dusty days from non-dusty days by thresholding daily AOD time series.

Otsu’s thresholding does not consider the labels of pixel values (DSE points) to differentiate two classes, so we calculated the WA of two randomly selected pixels including dusty days and non- dusty days as another thresholding method as follows:

TAOD=(AvAODdd×ndd)+(AvAODnd×nnd)N                                                    Eq. 1

Where,  TAOD indicates the AOD threshold to separate dusty days, AvAODdd and AvAODnd show average AOD for dusty days and non-dusty days, respectively. ndd and nnd show the number of selected pixels for dusty days and non-dusty days, respectively, and N indicates their sum.

Extract FIC of DSEs: The AOD threshold (TAOD) is considered the key parameter to extract the frequency (Equation 2), intensity (Equation 3), and continuity (Equation 5), as the main three indicators of dust events. In other words, annual AOD thresholds are used to separate daily dust pixels from non-dust pixels using daily AODs.

| Fdd=i=1kddi | Eq. 2 |
| --- | --- |

Where, Fdd is the sum of dusty days (days with AOD ≥ TAOD for a pixel in a specific time period). It is calculated annually from 2001 to 2022. ddi is the dusty day in ith day of the year.

| DDI=i=1kAODi-TAOD | Eq. 3 |
| --- | --- |

Where, DDI is the sum of differences between AOD and TAOD for the dusty days. DDI only exists for daily dust pixels.

|  | Eq. 4 |
| --- | --- |

Where the dust severity index (DSI) is the product of DDI and Fdd for dusty days in a certain pixel.

Finally, the continuity of dusty days (Cdd) was calculated using Equation 5 as follows:

Eq. 5

Where Cdd is defined as the continuity of dusty days for a pixel. nid and Cid show the number of consecutive dusty days, from 1 day to k days, respectively. The output Cdd demonstrates the average annual of continuous dusty days for a certain pixel.

- Results

  - Accuracy assessment of DSEs

To identify dusty from non-dusty days, the annual AOD thresholding using Otsu’s and WA was carried out. Daily AOD (2001-2022) were applied as inputs. 70% of DSEs annual hotspots were randomly selected and the best AOD threshold was assessed for each year. Then, the remaining 30% of dust hotspots were used to assess dusty days based on the assessed TAOD. Finally, the accuracy was measured by comparing the results with the dust hotspots. Hence, F1_score and overall accuracy (for all years, dusty years, and low dusty years) were calculated for assessed DSEs of AODs by Otsu’s thresholding and WA methods (Table 1).

Table 1. F1_score and overall accuracy of annual DSEs assessed by WA and Otsu’s thresholding for both AODs (blue and green).

|  | WA |  |  |  | Otsu’s thresholding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Selected years |  | Band | F1_score | overall accuracy |  | F1_score | overall accuracy |  |
| All years |  | Green | 0.83 | 0.73 |  | 0.71 | 0.62 |  |
|  |  | Blue | 0.75 | 0.63 |  | 0.69 | 0.58 |  |
| Dusty years |  | Green | 0.86 | 0.75 |  | 0.69 | 0.58 |  |
|  |  | Blue | 0.78 | 0.65 |  | 0.68 | 0.56 |  |
| Low dusty years |  | Green | 0.78 | 0.68 |  | 0.75 | 0.62 |  |
|  |  | Blue | 0.71 | 0.60 |  | 0.73 | 0.60 |  |

The accuracy assessment of AOD thresholding by Otsu and WA methods for both AODs (blue and green) revealed that the AOD thresholds assessed by WA for green AOD have higher accuracy. Fig. 3-a presents those threshold AODs with a range from 0.2 to 0.5 from 2001 to 2022. The average annual AOD map is shown in Fig. 3-b.  Determining the appropriate threshold for AOD is always controversial. This paper introduces the concept of determining the dynamic threshold for dust according to dust events. Here it is calculated as an annual threshold, and if required, it can be calculated on a monthly or daily basis.

Fig. 3. Assessed annual AOD thresholds by WA of green AOD in TEB (a) and average annual AOD map from 2001 to 2022 (b).

  - DSEs monitoring

The number of DSEs and the corresponding average of AODs were calculated annually from 2001 to 2022 (Fig. 4). Years of 2008, 2009, and 2022 with more than 1500 DSEs were identified as the highest dusty years, while 2002, 2014, 2019 and 2020 were identified as the least. The average AOD of DSEs ranges from 0.25 (2019) to 0.7 (2022). Although AOD values increase in years with more DSEs, the rise is not proportional to the number of dust events. As a result, AOD values alone cannot reflect the intensity and frequency of DSEs. Hence, extracting FIC indicators from the AOD time series could help to understand the spatiotemporal patterns of the DSEs regime better, which can be interpreted as a dust hazard.

Fig. 4. Annual number of DSEs and the corresponding average of AODs (2001 to 2022). AODs were multiplied by 1000 for easier comparison.

  - Frequency, intensity, and continuity of DSEs

The annual frequency of DSEs was calculated using Eq. 2 and the map of Average Fdd is shown in Fig. (5-a). The average annual dust severity index (DSI) of DSEs (Eq. 4) and average annual continuous dusty days (Eq. 5) are shown in Fig. 5-b and Fig. 5-c, respectively. The maps demonstrate a higher occurrence of DSEs in the central and southern regions of TEB compared to other areas. Severe DSEs have been observed in two central areas and three southeast areas (Fig. 5-b). Additionally, a higher concentration of continuous dusty days has been identified in the central and southern areas in the TEB (Fig. 5-c).

Daily DSIs of dust pixels were categorized using K-means clustering method (Abbas et al., 2016). In comparison to the defined TAOD, we differentiated the no-dust, low-dust, moderate-dust, and severe-dust pixels. Figures 5-d, 5-e, and 5-f display the maps representing the number of days with low, moderate, and severe DSEs from 2001 to 2022, respectively. Low and moderate DSEs are predominantly observed in the western regions of central lakes and wetlands, while severe dust events tend to occur in the eastern regions.

Fig. 5. The average annual maps of Fdd (a), DSI (b), continuous dusty days (c), number of days with low DSEs (d), moderate DSEs (e), and severe DSEs (f).

- Discussion

  - Dust clustering

The combination of dust severity clusters assessed by K-means is a supportive means for visual interpretation in dust event pattern studies. An RGB composite color representation of dust severity was produced by combining the average annual clusters of the dust severity (Fig. 6). This false-color composite map effectively illustrates the spatial distribution and density of dust severity, ranging from no dust to severe DSEs. Pixels exhibiting high proportions of red, green, and blue correspond to severe, moderate, and low DSEs, respectively. Conversely, pixels with a dark tone indicate low values across the red, green, and blue, representing regions with no dust. The created false color image depicting the severity of dust clusters reveals two distinct areas with severe DSEs. The first one is located in the east of the lakes and wetlands, while the second one is situated in the southeastern areas of the TEB. These areas of severe DSEs indicate that the dust hotspots in the eastern regions of the central lakes and wetlands exhibit a higher density compared to other dust source areas across the TEB.

Fig. 6. The RGB color map of 3 dust clusters including low, moderate, and severe dust pixels.

To comprehend and compare the annual trends of FIC indicators related to dust, six cities with populations exceeding one million people were selected for a more detailed analysis in the TEB. They include Gaziantep and Diyarbakir in Turkey and Mosul, Kirkuk, Baghdad, and Basra in Iraq (Fig. 6). Fig. 7 presents the annual frequency of DSEs and their severity, categorized into three clusters: low, moderate, and severe, for the selected cities. As depicted in Fig. 7-d and Fig. 7-b, Gaziantep and Basra recorded the lowest and highest annual frequencies of DSEs, respectively. Annual observations have indicated a high severity of dust events in Baghdad and Basra. However, considering the population of Baghdad, which exceeds 7 million people (Ali and Al Ramahi, 2020), the number of severe dusty days could potentially increase due to air pollution (Rabee, 2015). The northern cities, including Gaziantep, Diyarbakir, Mosul, and Kirkuk, recorded their maximum frequency of dust events in 2001. In contrast, Baghdad and Basra, located in the south of TEB, experienced their peak dust events in 2005 and 2008, respectively. The year 2022 was predominantly dusty in TEB (Fig. 4), with observations primarily confined to Basra due to the proximity of active dust hotspots. Fig.8 delineates four annual clusters of dust continuity for the selected cities spanning the period from 2001 to 2022. These clusters, which represent the duration of continuous dust events, are categorized into 1-day, 2-3 days, 4-6 days, and more than 6 days. Gaziantep and Diyarbakir experienced the highest proportion of 1-day DSEs, while Baghdad and Basra exhibited the highest percentage of the most continuous DSEs (those lasting 6 days or more). Baghdad and Kirkuk consistently observed DSEs with a duration of 4-6 days throughout the 22 years of the study. In contrast, Basra, despite having the highest dust frequency, only experienced 1-day and 1-3 days continuous dust events in the years 2005 and 2014, respectively.

| - Baghdad | d- Gaziantep |
| --- | --- |
| b- Basra | e- Kirkuk |
| c- Diyarbakir | f- Mosul |

Fig. 7.  The frequency and severity clusters of dusty days in Baghdad (a), Basra (b), Diyarbakir (c), Gaziantep (d), Kirkuk (e), and Mosul (f) from 2001 to 2022. The sum of 3 clusters for each year shows the frequency of dusty days (Fdd).

| a- Baghdad | d- Gaziantep |
| --- | --- |
| b- Basra | e- Kirkuk |
| c- Diyarbakir | f- Mosul |

Fig. 8. The frequency of continuous dusty days clusters in Baghdad (a), Basra (b), Diyarbakir (c), Gaziantep (d), Kirkuk (e), and Mosul (f) from 2001 to 2022

  - Dust probability distribution

From the statistical viewpoint, any dust storm can be studied as an independent and discrete probabilistic event. Consequently, the probability distribution of DSEs can be modeled using Poisson probability distribution (Equation 6). This distribution is typically employed to calculate the probability of a given number of events or random variables occurring over a fixed time interval (Cohen, 1960; Mighell, 1999).

Eq. 6

E(x)= ,   Var(x)=

Where fx(x=x) represents the probability of ‘x’ occurrence and the term λ is defined as the average number of occurrences within a specific period.

The probability distribution of DSE frequency, severe and continuous dusty days, was computed using the Poisson distribution for daily dust pixels. The outcomes of this dust probability distribution for the selected cities are depicted in Fig. 9. The plots of the probability distribution illustrate the probability value (ranging between 0 and 1) for the occurrence of each specific DSE frequency (Fig. 9-a and Fig. 9-b), severe dusty days (Fig. 9-c and Fig. 9-d), and continuous dusty days (Fig. 9-e and Fig. 9-f). The peak values of the distribution signify the average probability. The stretched shape of the distribution suggests a wider expected range of DSE frequencies, while a narrower and taller shape indicates a more limited range of DSE frequencies for a given location. Consequently, we forecast the annual DSE frequencies to be between 5 to 35 for Gaziantep and 60 to 130 for Basra, respectively (Fig. 9-a). Other cities show a DSE frequency within these two ranges. The shapes of the probability distributions for severe and continuous dusty days across the selected cities are relatively similar to each other. That is why, they are interpreted in the same manner as the shapes of DSE frequencies. It is unlikely (a probability less than 0.04) for Gaziantep to experience a severe dust event within a year, while Basra is anticipated to experience three severe dust events annually. Furthermore, it is projected that the annual DSEs in Gaziantep and Basra will exhibit a continuity of 1-day and 2-days, respectively.

The distribution of accumulative probability for DSE frequency indicates that the maximum annual expected DSE numbers for Gaziantep and Basra are 35 and 130, respectively (Fig. 9- b). The likelihood of observing fewer than 70 DSEs in Basra is close to 0, while the probability of Gaziantep experiencing a year with a DSE number <= 10 is approximately 0.5 (Fig. 9-b). The accumulative probability of severe dusty days in Gaziantep shows 0.95 for a year without severe DSE, then it is represented by a horizontal line with a constant value of 1, indicating that the probability of experiencing a maximum of one severe dust event is the same as experiencing a two or more. The accumulative probability of severe DSEs for other cities displays an ascending trend (Fig. 9-d). The accumulative probability of continuous DSEs for all cities is also ascending, with the slope of the curves being a distinctive feature. For instance, the probability that DSEs will last a maximum 3 days in Basra and Gaziantep is approximately 0.85 and 0.96, respectively (Fig. 9-f).

| - Probability of DSEs frequency =k | b- Accumulative probability of DSEs frequency <=k |
| --- | --- |
| c- Probability of severe dusty days =k | d- The accumulative probability of severe dusty days <=k |
| e- Probability of continuous dusty days =k | f- Accumulative probability of continuous dusty days <=k |

Fig. 9. The probability distributions and accumulative probability distributions of DSE frequency (a and b), severe dusty days (c and d), and continuous dusty days (e and f) for the selected cities.

The probability distribution curve of dust frequency exhibits a normal distribution (compared to the probability distribution of severe and continuous dusty days). Hence, the rules of normal distribution can be used to analyze different DSE frequencies. In the normal distribution curve, three distinct probability or confidence levels have been identified, namely 68%, 95%, and 99.7%.  Consequently, we provided the expected ranges of annual DSEs occurring in the selected cities within confidence levels 68%, 95%, and 99.7% in Table 2. The ranges of DSE numbers within the probability levels for Gaziantep and Basra are expected to be 14 to 22, 10 to 26, 6 to 32 and 83 to 101, 74 to 110, 65 to 119, respectively. The probability levels for the remaining cities exhibit varieties that lie between those of Gaziantep and Basra (Table 2). In general, the application of the probability distribution of DSEs offers valuable insights into the dust hazard conditions prevalent in these cities. This information can be instrumental in informing policies and strategies aimed at mitigating the substantial environmental and health impacts associated with dust storms.

Table 2. The expected ranges of annual DSEs occurring in the studied cities within the confidence levels 68%, 95%, and 99.7%

| City | Confidence level |  |  |
| --- | --- | --- | --- |
|  | 68% | 95% | 99.7% |
| Baghdad | 56-70 | 49-77 | 42-84 |
| Basra | 83- 101 | 74-110 | 65-119 |
| Diyarbakir | 17-25 | 12-30 | 8-34 |
| Gaziantep | 14-22 | 10-26 | 6-32 |
| Kirkuk | 35-47 | 28-54 | 22-60 |
| Mosul | 30-42 | 24-48 | 18-54 |

  - Dust hazard index

Hazard term denotes to the probability of occurrence, within a specified period in each area, of a potentially damaging natural phenomenon (Wilhite, 2000). Hazard dust is regarded as one of the components of dust risk analysis. Since the frequency (Eq. 2), intensity (Eq. 3), and continuity of dust events (Eq. 5) describe measures of dust hazard, they could combine to serve as the dust hazard index (DHI). DHI can be calculated through three spatial overlay methods including Sum, Product, and Hedged Product as Eq.s 7-9:

DHISum= Fdd+DSI+Cdd                                                                                    Eq. 7

DHIProduct= Fdd×DSI×Cdd                                                                                Eq. 8

DHIHP= Fdd×DSI×Cdd                                                                                   Eq. 9

Where, DHISum,  DHIProduct and  DHIHP indicate dust hazard index calculated by Sum, Product, and Hedged Product, respectively. The average annual frequency, intensity, and continuity of dusts were normalized and used as inputs of DHI. The range of the final DHI map varies from 0 to 1, where regions with higher values indicate greater dust.

Output DHIs obtained by Sum and Product decrease and increase the difference between the maximum and minimum values, respectively. The Hedged Product shows the differences in a more balanced way (Fig. 10). Hence, due to the flexible and nonlinear impact of the Hedged Product (Boloorani et al., 2023a), it is recommended for dust hazard analysis. The dust hazard index map of the TEB was produced by the Hedged Product (Fig. 11). Areas around wetlands and southern regions are recognized as areas with high dust hazards during the last two decades (2001 to 2021). In general, the dust hazard belt in the TEB starts from the central regions and moves downward in the northwest-southeast direction covering the southern regions.

Fig. 10. Three methods of combining the annual FIC layers of dust events to calculate the Dust Hazard Index (DHI) and compare the resulted values ​​in the studied cities.

Fig. 11. The dust hazard index (DHI) map calculated using the dust frequency, dust intensity, and dust continuity (using the Hedged Product method).

- Conclusion

An increase in AOD is observed within the TEB in years with more occurrences of DSEs from 2001 to 2022. However, this increase does not exhibit a direct proportionality with the frequency and intensity of dust events. Therefore, the extraction of frequency, intensity, and continuity of DSEs from AOD time series data enhanced our understanding of the spatiotemporal patterns of dust events. This study conducted an exhaustive examination of MODIS AOD time series within the TEB region and yielded significant insights into the frequency, intensity, and continuity of dust storm events. The study identified annual AOD thresholds using the weighting average method based on daily AOD and dust hotspots datasets, establishing a robust framework for distinguishing between dusty and non-dusty days. The spatiotemporal behavior of DSEs, as revealed by clustering and probability distribution models, identified the central and southern regions of the TEB as critical dust hotspots. The DHI further accentuates these findings, offering a quantitative measure of dust hazard across the basin. These results not only contribute to the scientific understanding of dust storm dynamics in arid and semi-arid regions, but also provide a basis for developing dust control and mitigation policies, and ultimately aim to reduce the adverse environmental and health impacts of dust storms in the TEB and similar regions. Future work should focus on integrating more meteorological data and exploring the socio-economic impacts of dust storms to enhance the comprehensiveness of dust hazard assessments.

References

Abbas, A. W., Minallh, N., Ahmad, N., Abid, S. A. R., & Khan, M. A. A. (2016). K-Means and ISODATA clustering algorithms for landcover classification using remote sensing. Sindh University Research Journal-SURJ (Science Series), 48(2).

Aghababaeian, H., Ostadtaghizadeh, A., Ardalan, A., Asgary, A., Akbary, M., Yekaninejad, M. S., & Stephens, C. (2021). Global health impacts of dust storms: a systematic review. Environmental health insights, 15, 11786302211018390.

Ali, A. K. M., & Al Ramahi, F. K. M. (2020). A study of the effect of urbanization on annual evaporation rates in Baghdad city using remote sensing. Iraqi Journal of Science, 2142-2149.

Al-Ansari, N. (2016). Hydro-politics of the Tigris and Euphrates basins. Engineering, 8(3), 140-172.

Al-Hemoud, A., Al-Dashti, H., Al-Saleh, A., Petrov, P., Malek, M., Elhamoud, E., ... & Middleton, N. (2022). Dust storm ‘hot spots’ and transport pathways affecting the Arabian Peninsula. Journal of Atmospheric and Solar-Terrestrial Physics, 238, 105932.

Al-Hemoud, A., Al-Dousari, A., Al-Dashti, H., Petrov, P., Al-Saleh, A., Al-Khafaji, S., ... & Koutrakis, P. (2020). Sand and dust storm trajectories from Iraq Mesopotamian flood plain to Kuwait. Science of the total Environment, 710, 136291.

Al-Hemoud, A., Al-Dousari, A., Misak, R., Al-Sudairawi, M., Naseeb, A., Al-Dashti, H., & Al-Dousari, N. (2019). Economic impact and risk assessment of sand and dust storms (SDS) on the oil and gas industry in Kuwait. Sustainability, 11(1), 200.

Bakhtiari, M., Darvishi Bolorani, A., Abdollahi Kakroodi, A., & Rangzan, K. (2018). The trending of normalized health environmental index based on satellite data from 2001 to 2013 and its relation with dust sources in the West Asia. Scientific-Research Quarterly of Geographical Data (SEPEHR), 27(106), 87-99.

Bakhtiari, M., Boloorani, A. D., Kakroodi, A. A., Rangzan, K., & Mousivand, A. (2021). Land degradation modeling of dust storm sources using MODIS and meteorological time series data. Journal of Arid Environments, 190, 104507.

Beyranvand, A., Azizi, G., Alizadeh, O., & Darvishi Boloorani, A. (2023). Dust in Western Iran: the emergence of new sources in response to shrinking water bodies. Scientific Reports, 13(1), 16158.

Boloorani, A. D., Kazemi, Y., Sadeghi, A., Shorabeh, S. N., & Argany, M. (2020). Identification of dust sources using long term satellite and climatic data: A case study of Tigris and Euphrates basin. Atmospheric Environment, 224, 117299.

Boloorani, A. D., Papi, R., Soleimani, M., Karami, L., Amiri, F., & Samany, N. N. (2021). Water bodies changes in Tigris and Euphrates basin has impacted dust storms phenomena. Aeolian Research, 50, 100698.

Boloorani, A. D., Bakhtiari, M., Samany, N. N., Papi, R., Soleimani, M., Mirzaei, S., & Bahrami, H. A. (2023a). Land degradability mapping using remote sensing data and soil chemical properties. Remote Sensing Applications: Society and Environment, 32, 101027.

Boloorani, A.D., Papi, R., Soleimani, M., Al-Hemoud, A., Amiri, F., Karami, L., Samany, N.N., Bakhtiari, M. and Mirzaei, S. (2023b). Visual interpretation of satellite imagery for hotspot dust sources identification. Remote Sensing Applications: Society and Environment, 29, p.100888.

Darvishi Boloorani, A., Soleimani, M., Papi, R., Neysani Samany, N., Teymouri, P., & Soleimani, Z. (2023c). Sources, drivers, and impacts of sand and dust storms: A global view. In Dust and Health: Challenges and Solutions (pp. 31-49). Cham: Springer International Publishing.

Cohen, A. C. (1960). Estimating the parameter in a conditional Poisson distribution. Biometrics, 16(2), 203-211.

Deng, J., Wang, T., Jiang, Z., Xie, M., Zhang, R., Huang, X., & Zhu, J. (2011). Characterization of visibility and its affecting factors over Nanjing, China. Atmospheric Research, 101(3), 681-691.

Furman, H. K. H. (2003). Dust storms in the Middle East: sources of origin and their temporal characteristics. Indoor and Built Environment, 12(6), 419-426.

Ginoux, P., Chin, M., Tegen, I., Prospero, J. M., Holben, B., Dubovik, O., & Lin, S. J. (2001). Sources and distributions of dust aerosols simulated with the GOCART model. Journal of Geophysical Research: Atmospheres, 106(D17), 20255-20273.

Gupta, P., Remer, L. A., Levy, R. C., & Mattoo, S. (2018). Validation of MODIS 3 km land aerosol optical depth from NASA's EOS Terra and Aqua missions. Atmospheric Measurement Techniques, 11(5), 3145-3159.

Guo, S. J., & Chen, M. (2016). Pollution characteristic of atmospheric carbonyls during one haze event in Nanning, South China. Aerosol and Air Quality Research, 16(5), 1143-1151.

Hamidi, M., Kavianpour, M. R., & Shao, Y. (2013). Synoptic analysis of dust storms in the Middle East. Asia-Pacific Journal of atmospheric sciences, 49, 279-286.

Jebali, A., Zare, M., Ekhtesasi, M. R., & Jafari, R. (2021). A new threshold free dust storm detection index based on MODIS reflectance and thermal bands. GIScience & Remote Sensing, 58(8), 1369-1394.

Kaskaoutis, D. G., Kambezidis, H. D., Nastos, P. T., & Kosmopoulos, P. G. (2008). Study on an intense dust storm over Greece. Atmospheric Environment, 42(29), 6884-6896.

Liu, Z., Liu, Q., Lin, H. C., Schwartz, C. S., Lee, Y. H., & Wang, T. (2011). Three‐dimensional variational assimilation of MODIS aerosol optical depth: Implementation and application to a dust storm over East Asia. Journal of Geophysical Research: Atmospheres, 116(D23).

McTainsh, G., Chan, Y. C., McGowan, H., Leys, J., & Tews, K. (2005). The 23rd October 2002 dust storm in eastern Australia: characteristics and meteorological conditions. Atmospheric Environment, 39(7), 1227-1236.

Marchese, F., Sannazzaro, F., Falconieri, A., Filizzola, C., Pergola, N., & Tramutoli, V. (2017). An enhanced satellite-based algorithm for detecting and tracking dust outbreaks by means of SEVIRI data. Remote Sensing, 9(6), 537.

Mehrabi, S., Soltani, S., & Jafari, R. (2015). Analyzing the relationship between dust storm occurrence and climatic parameters. Journal of Science and Technology of Agriculture and Natural Resources, 19(71), 69-81.

Middleton, N. J. (2017). Desert dust hazards: A global review. Aeolian research, 24, 53-63.

Middleton, N. J., & Goudie, A. S. (2001). Saharan dust: sources and trajectories. Transactions of the Institute of British Geographers, 26(2), 165-181.

Mighell, K. J. (1999). Parameter estimation in astronomy with poisson-distributed data. I. the χγ2 statistic. The Astrophysical Journal, 518(1), 380.

Naghibi, A., Hashemi, H., Zhao, P., Brogaard, S., Eklund, L., Hassan, H. H., & Mansourian, A. (2024). Spatiotemporal variability of dust storm source susceptibility during wet and dry periods: The Tigris-Euphrates River Basin. Atmospheric Pollution Research, 15(1), 101953.

Namdari, S., Karimi, N., Sorooshian, A., Mohammadi, G., & Sehatkashani, S. (2018). Impacts of climate and synoptic fluctuations on dust storm activity over the Middle East. Atmospheric environment, 173, 265-276.

Prasad, A. K., & Singh, R. P. (2007). Changes in aerosol parameters during major dust storm events (2001–2005) over the Indo‐Gangetic Plains using AERONET and MODIS data. Journal of Geophysical Research: Atmospheres, 112(D9).

Qian, W., Quan, L., & Shi, S. (2002). Variations of the dust storm in China and its climatic control. Journal of climate, 15(10), 1216-1229.

Romano, F., Ricciardelli, E., Cimini, D., Paola, F. D., & Viggiano, M. (2013). Dust Detection and Optical Depth Retrieval Using MSG-SEVIRI Data. Atmosphere, 4(1), 35-47.

Rabee, A. M. (2015). Estimating the health risks associated with air pollution in Baghdad City, Iraq. Environmental monitoring and assessment, 187, 1-12.

Soleimani, Z., Teymouri, P., Boloorani, A. D., Mesdaghinia, A., Middleton, N., & Griffin, D. W. (2020). An overview of bioaerosol load and health impacts associated with dust storms: A focus on the Middle East. Atmospheric Environment, 223, 117187.

Song, Q., Zhang, Z., Yu, H., Ginoux, P., & Shen, J. (2021). Global dust optical depth climatology derived from CALIOP and MODIS aerosol retrievals on decadal timescales: regional and interannual variability. Atmospheric Chemistry and Physics, 21(17), 13369-13395.

Sotoudeheian, S., Salim, R., & Arhami, M. (2016). Impact of Middle Eastern dust sources on PM10 in Iran: Highlighting the impact of Tigris‐Euphrates basin sources and Lake Urmia desiccation. Journal of Geophysical Research: Atmospheres, 121(23), 14-018.

Tripathi, S. N., Dey, S., Chandel, A., Srivastava, S., Singh, R. P., & Holben, B. N. (2005, June). Comparison of MODIS and AERONET derived aerosol optical depth over the Ganga Basin, India. In Annales Geophysicae (Vol. 23, No. 4, pp. 1093-1101). Göttingen, Germany: Copernicus Publications.

Wei, J., Li, Z., Peng, Y., & Sun, L. (2019). MODIS Collection 6.1 aerosol optical depth products over land and ocean: validation and comparison. Atmospheric Environment, 201, 428-440.

Wilhite, D. A. (2000). Drought planning and risk assessment: Status and future directions. Annals of Arid Zone, 39(3), 211-230.

Wu, L. (2007). Impact of Saharan air layer on hurricane peak intensity. Geophysical research letters, 34(9).

Xie, Y., Zhang, W., & Qu, J. J. (2017). Detection of Asian dust storm using MODIS measurements. Remote Sensing, 9(8), 869.

Xu, X., Xu, S., Jin, L., & Song, E. (2011). Characteristic analysis of Otsu threshold and its applications. Pattern recognition letters, 32(7), 956-961.

Yang, P., Song, W., Zhao, X., Zheng, R., & Qingge, L. (2020). An improved Otsu threshold segmentation algorithm. International Journal of Computational Science and Engineering, 22(1), 146-153.